PART A: INFORMATION FOR THE TENDERER

Name and address of the contracting authority: General Hospital ”Đorđe Joanović” Zrenjanin, Dr Vase Savića 5, 23000 Zrenjanin Serbia Title of the tender: IT Equipment Reference number: RORS00040/GHZr/TD7 Date of launching: 08/12/2025

- INFORMATION ON SUBMISSION OF THE TENDERS

Subject of the contract:

The subject of this tender is:

- Provision of supplies as indicated in the technical information in the point 2 of these information;

Deadline for submission of the tenders:

The deadline for submission of tenders is 17/12/2025 at 12:00 hours. Any tender received after this deadline will be automatically rejected.

The Contracting Authority will provide answer to all questions received no latter then 3 days before the deadline and has to publish them on the same web sites where the tender was published. Questions regarding this procedure may be submitted to milena.maksimovic77@gmail.com

until 11/12/2025 at 24.00h.

Currency

Tenders must be presented in EUR or RSD with VAT included.

Note:

In accordance with IPA implementing regulation, for all partners VAT can be an eligible expenditure. The Contracting Authority will pay the unit prices, as stated in the Financial Offer, and will pay the VAT if the VAT is eligible and is clearly identified on the invoices.

For this contract VAT is eligible cost.

In case when the offers are submitted in RSD, the exchange rate to be used for checking financial compliance with available budget (during financial evaluation), shall be InforEuro exchange rate for the month when the tender is launched

Award criteria:

- The sole award criterion will be the price. The contract will be awarded to the lowest compliant tender.

In case that one offer is received, the Contracting Authority shall check whether the offer is administratively and technically compliant with the requirements set by this tender documentation and financial offer is within the available budget.

Variant solutions will not be taken into consideration.

Award notification

The successful tenderer will be informed of the results of the evaluation procedure in written form.

Contract award notice will be published on the programme website. The estimated time of communication publishing of results to the tenderers is 7 days after the date of contract signature.

Address and meanings for submission of the tenders:

The tenderers will submit their tenders using the standard submission form available in the “Part B: Format of offer to be provided by the tenderer” of the tender dossier. The tender will be submitted in 1 original. Any tenders not using the prescribed form may be rejected by the contracting authority.

In addition to the offer the tenderer is required to provide the following supporting documentation:

- Copy of legal registration (only if  not publicly available from the Serbian Business Register Agency website)

- Proof of delivery under at least one supply contract involving similar equipment with a value no less than 19.200 EUR (with VAT included) delivered in following timeframe: within 5 years prior submission date

The tenders will be submitted in a sealed envelope, containing the following information:

- Name and address of the tenderer

- Title of the tender: IT Equipment

- Reference number: RORS00040/GHZr/TD7

- The words: ‘’Not to be opened before the tender opening session’’ and ‘’Ne otvarati pre sastanka za otvaranje ponuda”

The tenders will be submitted in person, by post or courier service to the following address:

General Hospital ”Đorđe Joanović” Zrenjanin, Dr Vase Savića 5, 23000 Zrenjanin Serbia

For: Director

The tenderers are reminded that in order to be eligible the tenders need to be received by the Contracting Authority by the deadline indicated above.

- TECHNICAL INFORMATION

The tenderers are required to provide supplies as indicated below. In the tenderer’s technical offer, the tenderers might indicate more details on the deliveries, including brand names if applicable, referring back to the below table.

| No. | Number of items | Title of item | Technical specifications |
| --- | --- | --- | --- |
| Supplies |  |  |  |
| 1.1. | 1pc | TV Set 55” | TV Set 55” - Minimum screen size: 55 inch (eq ~ 139cm) - Screen resolution: min 3840x2160 (UltraHD ~ 4K) - Screen refreshment rate: min 60Hz - Minimum required connectivity/number of ports: 1xUSB, 3xHDMI, 3.5mm audio out, LAN, WiFi |
| 1.2. | 1pc | Server | Server - Server type: Rack mount 1U with rails - CPU ~ min 4 cores, min 12 threads, total cache min 12MB, Thermal design power less than 70W, max frequency min. 4,8GHz - RAM ~ min 32GB DDR4 ECC or better, expandable to 128GB - HDD: min 3x0.9TB hot-plug or better; min 4 drive bays; min 4x NVMe PCIe SSD supported; internal DVDRW/Blu-ray drive supported - Power supply: min 500W hot-plug, optinal 2nd for redundancy - Interfaces: 5× USB 3 of which 1 min 20 GBit/s, 2× USB 2, 1× VGA;optional additional 1× front VGA, 1× RS-232, 2× M.2 - Cooling system: min 5 coolers - Network connectivity: 2× 10/100/1000 Mbit/s Ethernet (TCP/IP acceleration functionality), 1x remote management port |
| 1.3. | 5pcs | Desktop PC | Desktop PC - CPU: min 10 cores, min 16 threads, - CPU max frequency min. 4,8GHz,: min 3.6GHz - Total CPU cache memory min: 24Mb - CPU base power: max 45W - RAM: min 16GB DDR4, - HDD: min 1Tb M.2 NVMe or better - Monitor: min size 27", resolution Full HD (1920x1080) LED, with HDMI & VGA connection, refresh rate min 100Hz, response time max 1msec, with integrated sound speakers - Aux: keyboard with YU keys, optical mouse, all necessary cables. - Operative system: Win11 pro or equivalent |
| 1.4 | 5pcs | Laser Printer monochrome | Laser Printer monochrome - Printer type: laser monochrome A4 - Max print speed A4 : min 40pgs/min - First page print time: max 6,5 sec - Print quality: min 1200dpi - Input paper box capacity: min 100sheets - Operative system support for: Win11 or equivalent - Connectivity: USB, LAN, USB cable included - Capable to print at least 3000 pages in line with factory specifications with one toner cartridge - One full toner cartridge included |
| 1.5 | 1 | Laser printer color | Laser printer color - A3 color laser multifunctional device 3-in-1 (printer, scanner, copier) - Print speed / copying ≥ 26 ppm A4 i ≥ 15 ppm A3 - Print resolution ≥ 1200 x 600tpi - First copy time ≤ 6,1s/8,4s mono/color - Memory ≥ 4,5GB + 256GB SSD HDD - Automatic duplex printing - USB printing formats: PDF, EPS, TIFF/JPEG i XPS - ADF capacity ≥ 100 sheets - Scanning resolution 600 x 600 dpi - Scanning speed ≥ 35ipm/70ipm single side / double side - Scan formats: TIFF, JPEG, PDF, XPS, Office Open XML - Multipurpose paper tray ≥ 100 sheets A4 - Supported media weights 52 do 300 g/m2 - Paper Supply Capacity (A4, 80 gsm) ≥ 550 sheet A4 x 2 - Connectivity 1000Base-T/100Base-TX/10Base-T, USB 2.0 (host) x1, USB 3.0 (host) x1, USB 2.0 device x1 - Display ≥ 10,1" touch sensitive - Consumables capacity: the device must be supplied with a 2 sets of toner with a minimum capacity of 25,000 prints per color - Typical Electricity Consumption (TEC) ≤ 0.26 kWh - Pedestal type with wheels |
| 1.6 | 1pc | Laptop PC | Laptop PC - Display size: 15.6" - Display resolution: 1920x1200 - CPU – min 6 cores, min 12 threads, total cache min 13MB, Thermal design power max 45W, max frequency min.4.3GHz - RAM – min 16GB - HDD: min 500GB NVME or better - Onboard camera: min 720p - Interfaces: min 1HDMI, min 3 USB, min 1 3.5mm audio out - Software installed: Win11 pro or equivalent |
| 1.7 | 1set | IT equipment for online Training | IT equipment for online Training - Microphone 20pcs – self-standing; 3,5mm jack, cable length 1,5m-2m - Sound speaker 20pcs - USB interface, 3,5mm jack output, cable length 120-150cm; RMS min 12W - Headset 20pcs - wired 3,5mm jack, cable length min 150cm with volume control - LCD Laser Projector Transparent LCD panel (x 3, R/G/B) Panel size: 16.3 mm (0.64 in) diagonal (16:10 aspect ratio) Number of pixels: 2,304,000 (1920 x 1200) pixels Light source: Laser diodes Light output: 5,200 lm Time until light output declines: NORMAL mode 20,000 hours, 24,000 hours [ECO] Contrast Ratio: 3,000,000:1 Screen size (diagonal): 0.76–7.62 m (30–300 in), 16:10 aspect ratio Lens: 1.6x manual zoom (throw ratio: 1.09–1.77:1), manual focus lens, F 1.60–2.12, f 15.30–24.64 mm Lens shift Vertical: (from center of screen) +44 % Lens shift Horizontal: (from center of screen) ±20 % Keystone correction range Vertical: ±25 °, Horizontal: ±35 ° Terminals: HDMI IN (19-pin x 2 (Compatible with HDCP, Deep Color, 4K/30p*6 signal input), CEC Supported), D-sub 15-pin (female) x 2 (RGB/YP P /YC C ), Audio IN M3 stereo mini-jack x 2, Audio Out M3 stereo mini-jack x 1, Serial D-sub 9-pin, LAN RJ-45 x 1 for network control, 10Base-T, 100Base-TX, compatible with PJLink, USB connector (Type A) x 1 Maximum power consumption: 300 W (3.1–1.3 A) (305 VA)(Power consumption is 290 W at 200–240 V) Built-in speaker:10 W monaural Operation noise: 36 dB [NORMAL], 36 dB [ECO], 26 dB [QUIET] Dimensions: (W x H x D): 399 mm x 115 mm x 348 mm (15 23/32˝ x 4 17/32˝ x 13 11/16˝ ) (not including protruding parts) Weight: Approx. 6.5 kg Body Colour: White Or better - HDMI optical cable 20m HDMI 2.1 Active optical cable – HDMI A male - HDMI A male wit Interchangeable connector Resolutions up to 8K (7680 x 4320p) 4:4:4 at 60 Hz Bandwidth of 48 Gbps via OM3 optical fiber technology Backward compatible with HDMI 2.0 and HDMI 1.4 Transmission encoding :Uncompressed AOC End to end latency: 10 µs Maximum transmission bit rate: 48 Gbps (4-core – 12 Gbps per core) Audio formats: 2 Ch PCM, Multichannel LPCM, DTS-X, Dolby Atmos Communication and control : HDCP Pass-through, 2.3 Supported, EDID Pass-through, RX > TX, Ethernet 100 Mbps supported, (CEC) Bidirectional supported. Power supply: Passive, 4.5V ~ 5.5V VCC (from connected source) Max power consumption: 250 mW Or better - Projector screen Electric, control at the touch of a button, screen stop position with roller mechanism Weighted rod at base Suitable for wall or ceiling mounting Optional extension brackets Wide format (16:9) 300 cm |
| 1.8 | 1pc | Monitor for Education 75" | Monitor for Education 75" - Minimum screen size: 75 inch (eq ~190,5cm) - Screen resolution: min 3840x2160 ( UltraHD – 4K) - Screen refreshment rate: min 50 Hz - Audio: min 20W - Minimum required connectivity/ number of ports:2xUSB, 3xHDMI, 3.5mm audio out, LAN, WiFi, Bluetooth - Operative system: supported e.g. smart TV, with web browser - 2 years warranty |

NOT TO BE FILED IN BEFORE CONTRACT SIGNING NOT TO BE SUBMITTED WITHIN THE OFFER!!!

FORMAT OF THE CONTRACT BETWEEN THE CONTRACTOR AND THE CONTRACTING AUTHORITY

CONTRACT TITLE: IT Equipment

REF: RORS00040/GHZr/TD7

Concluded between:

General Hospital ”Đorđe Joanović” Zrenjanin,

Dr Vase Savića 5, 23000 Zrenjanin Serbia

(Contracting Authority)

AND

<Title>

<Address of the contractor>

Official registration number/ VAT number>

(Contractor)

Article 1: Subject of the contract

The subject of the contract is the supplies as indicated in the contractor’s offer – ‘’Part B: Format of offer to be provided by the tenderer’’.

Article 2: Contract value

The total contract value for implementation of delivery of supplies indicated in the Article 1 is: <XXX EUR/RSD> with VAT included.

In accordance with IPA implementing regulation, for all partners VAT can be an eligible expenditure. The Contracting Authority will pay the unit prices, as stated in the Financial Offer, and will pay the VAT if the VAT is eligible and is clearly identified on the invoices.

For this contract VAT is eligible cost.

Article 3: Contracting documents

The documents which form the part of this contract are (by the order of precedence):

- Contract agreement

- Contractor’s offer as provided in the tendering phase – ‘’Part B: Format of offer to be provided by the tenderer’’

- Any other supporting documentation if applicable

For any issues not defined in this contract agreement the rules of General conditions will be applied

| C4e | Draft contract : General conditions (Annex I) | https://wikis.ec.europa.eu/download/attachments/44169042/c4e_annexigc_en.pdf |
| --- | --- | --- |

https://wikis.ec.europa.eu/display/ExactExternalWiki/Annexes#Annexes-AnnexesC(Ch.4):Supplies

Article 4: Deliveries and payments

The contractor will deliver without reservation the supplies indicated in the contractor’s offer ‘’Part B: Format of offer to be provided by the tenderer’’. The deliveries will be implemented within the indicated dates.

The Contracting Authority will pay to the contractor the supplies in the amount indicated in the Article 2 of this contract document.

In case the contract is concluded in EURO, and payments are made in RSD, applicable exchange rate must be InforEuro exchange rate for the month of the issuing of invoice.

The payments will be issued by the following time schedule.

| Month |  | <EUR/RSD> |
| --- | --- | --- |
| 3 | Balance final payment | < 100 % of the contract value / Absolute amount > |
|  | Total | <Total contract value> |

Article 5: Duration of the contract

The duration of the contract is 3 months.

Commencement date is date of signature of contract by both parties.

Article 6: Resolving of disputes

Any disputes arising out of or relating to this Contract which cannot be settled otherwise shall be referred to the exclusive jurisdiction of relevant Serbian Court of Law in accordance with the national legislation of the state of the Contracting Authority.

| For the Contractor |  | For the Contracting Authority |  |
| --- | --- | --- | --- |
| Name: |  | Name: |  |
| Title: |  | Title: |  |
| Signature: |  | Signature: |  |
| Date: |  | Date: |  |