- ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Fire engine and Firefighting equipment for project PREVENT	p 1 /…

Publication reference: RORS00268 DVD-2/L2

LOT 2 - Firefighter equipment

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | FIREFIGHTER EQUIPMENT |  |  |  |
|  | ELECTRIC BREAKER – 1 pcs. - Battery tool combined for cutting and splitting - Battery-powered rescue cylinder - Technical characteristics: - Battery tool combined for cutting and splitting: 1 pcs. - Spreading width: min. 360 mm - Speading force (in the working area): 35-783 kN - Opening width: min. 290mm - Pulling width (with adapters): min. 400 mm - Max. Pulling force: 57kN - Weight (ready for use): max. 20 Kg - Dimensions (LxWxH): max. 940x240x250 mm - Nominal pressure: max. 700 bar - Battery: 2 pcs., 28V/5Ah M28B5 - Power supply 230 V for one battery: 1 pcs - Charger: 28V 220/240V 50/60 Hz - Technical characteristics: - Battery-powered rescue cylinder: 1 pcs. - Closed length (retracted): max. 540mm - Final length (extended): min. 900 mm - Stroke: 350-380 mm - Force: min. 110 kN - Weight (ready for us): max. 20 kg - Dimensions (LxWxH): max. 550x140x350mm - Nominal pressure: min. 700 bar - Battery: 2 pcs - 28V/5Ah M28B5 - Power supply 230 V for one battery: 1 pcs - Charger 28V 220/240V 50/60 Hz |  |  |  |
|  | BREATHING APPARATUS 6.8L WITH COMPOSITE BOTTLE – 1 pcs. - Technical characteristics: - Weight of complete set withface mask, lung demand valve and cylinder: max. 12 Kg - Input pressure (bar): 0-300 bar - Normal 1st stage output pressure: 7.5 - 1st stage output flow (l/min): >1,000 - Pneumatic warning whistle activation pressure: 50-60 - Pneumatic whistle sound Level (dBA): >90 - Operating temperature °C : - 30 °C to + 60 °C - Height-adjustment, a pivoting and sliding waist belt, and an easily adjustable harness to fit every shape and size of firefighter. |  |  |  |
|  | JET TURBO – 2 pcs. - Technical characteristics: - Type: C (fi 52) or equivalent - With slide valve - Adjustable flow rate in l/min approx: - 130 - 235 - 400 - Jet distance: minimum 40 m - Length: 30-40 cm - Weight: max. 3 kg - Operating pressure: min. 5 bar |  |  |  |
|  | FIREMAN'S BOOTS WITH LACES – 10 pcs. - Size and Quantity: EU42 – 3 pcs, 43 - 1 pc, 44 – 1 pc, 45 – 2 pcs, 46 – 3 pcs - Technical characteristics: - Firefighting leather boots, with zipper and laces, individual adjustment of 2 separate zones - The offered boots shall fulfill the following requirements: - Firefighting boots according to EN 15090:2012 - F2A HI3 CI AN SRC, Type 2, Class code I, Design „D“ or equivalent - Electrostatically dissipative shoes (ESD) tested according to EN 61340-4-3:2018 or equivalent - A flexible zone shall be located at the ankle area between shaft and foot - Reflective elements made of reflective material - Weight: max. 3 kg (1 pair) - Shaft height: 280-300 mm - Locking system & width adjustment - The boot shall have a zipper and laces (no push rotary-knob) - Heat and flame resistant straps shall be made of aramid or similar material - Insole - The insole shall be breathable and washable at 30 °C - The insole shall have an integrated cushioning in the heel area - Upper: leather - Colour: black - Smooth water-repellent, hydrophobic leather with reflective surface (reduces heating up in the sun) - Leather thickness: 2.1 mm – 2.3 mm - Ankle protection „ON“ - Special memory foam for maximum protection against impact in the ankle area - Toe protection & protection against perforation - The sole shall have an integrated steel anti-perforation protection - Toe protection cap made of steel - Steel plate in a horse shoe shape below the steel toe cap - Climate regulation system / membrane system / lining - Breathable shaft end by means of a mesh material |  |  |  |
|  | FIREMAN'S HELMET – 10 pcs. - Technical characteristics: - Size: One-size-fits-all; can be continuously adjusted to fit head sizes - 50 – 67 cm using the external adjusting wheel - Weight: max. 1.500 g in basic version with face guard, neck and ear cover - Helmet shell: Helmet shell made of composite material consisting of glass-fibre reinforced plastic (PA-GF) or similar, high temperature resistant, with circumferential helmet shell end rail - Helmet shell colours: fluorescent yellow (RAL 1026) - Customised marking and labelling: - Reflective strips flame resistant fabric. Additional reflective and design strips as well as functional markings made of thermally resistant, highly flexible, long-lasting and retro-reflective tape - Shock absorption system: Consisting of a lateral rigid foam element made of two-components polyurethane (PUR) reinforced with aramid fabric and a soft foam element at the crown and in combination with textile crown straps and a shock absorption pad - Material: Glass-fiber-reinforced plastic (PA-GF), high temperature resistant - Neck and ear cover: - The neck and ear cover is made of extremely flame retardant material. - Face guard: - Made of high temperature resistant material, with highest optical quality (optical class 1), continuously swivelling up and down; multiple coatings (permanent anti-fog coating inside and anti-scratch coating outside) – clear |  |  |  |
|  | FIREFIGHTER'S GLOVES – 10 pcs. - Size and Quantity: size 10 – 5 pcs, size 11 – 5 pcs - Technical characteristics (general): - Gloves shall be tested and certified according to EN 659:2003+A1:2008+AC:2009, or equivalent - Material (outer layer): - Back of the hand and cuff shall be made of high quality with a nonwoven laminated onto the backside for heat and cut protection - Palm side shall be made of silicon-carbon mineral coated -doubleface-fabric for excellent dexterity, high abrasion resistance and anti-slip on wet and smooth surfaces - Gloves shall be equipped with a watertight and breathable moisture barrier onto knitwear - All three layers in the glove shall be connected, therefore no becoming loose or slipping of inner material is possible when putting on or taking off the gloves - Outer fabric colour: dark blue or by the agreement - Shall be machine washable up to 60°C - Design: - Gloves shall be performed with a flexible wristlet adjustment - Pleats on the fingers shall ensure excellent movement - Gloves shall be equipped with PU impact protectors on the back of the hand - Gloves shall be designed with wristlet made of aramide or similar material - For connection of left and right gloves a hook and eye with a small carabine shall be mounted - Required performance level: - Abrasion resistance (EN 388): ≥ 4 - Cut resistance (EN 388): ≥ 4 - Tear resistance (EN 388): ≥ 4 - Puncture resistance (EN 388): ≥ 3 - Burning behaviour (EN 407): ≥ 4 - Dexterity (EN 420): ≥ 4 |  |  |  |
|  | UNDER HELMET – 10 pcs. - Technical characteristics: - Three-dimensional design (ergonomic cut) - Working with a flat seam (avoids pressure marks) - Large field of vision with elastic band (optimal fit in 3-point and 5-point breathing mask) - RFID (Electronic Identification System) - Ergonomically extended chest and neck area (longer cut with wider coverage that extends into the jacket) - Layer/Liner: knit, antistatic - Colour: Darknavy (203) - Norm/Standard: EN 13911:2017 or equivalent - Size: One size |  |  |  |
|  | FIRE SUIT – 10 pcs. - Size and Quantity: size 54 – 1 pc, 56 – 3 pcs, 58 – 2 pcs, 60 – 3 pcs, 62 – 1 pc - Technical characteristics: - Fire suit designed and intended to be worn during firefighting interventions in quality according to the specified standards, which determine the minimum levels of performance requirements for protective clothing that protects against heat and fire (level of thermal protection, protection against splashing of chemicals and flammable liquids, minimum performance levels of the materials used, level of visibility - Construction/Model/Design: - The suit consists of two parts: jacket and trousers - The jacket has a straight cut. The front fastens with a purpose-made zipper, which has a puller for quick separation (TOP OPEN with metal teeth). The zipper is covered with a protective strip. The blouse has a raised collar for better neck protection and fastens with a placket and velcro. A pocket is closed with a Velcro-fiber tape system. On the left side of the jacket, at chest height, there is a pocket with a strap for a tie. In the waist area, 2 pockets with a flap are sewn. The sleeves is regulated with a velcro-fiber tape system. Non-flammable reflective tapes in grey and yellow, are sewn on the jacket. A wide fiber strip is sewn on the back for a label that can be attached with velcro - Pants: - The trousers are a belt with a slightly raised rear part as protection for the kidneys. The suspenders are made of an elastic band about 4 cm wide, threaded through plastic regulators and fastened with durable plastic buckles. The pants fasten in the front with a spiral zipper - They are cut in the waist area, where the two slanted flaps are located, which are closed with a velcro-fiber tape system. In the side parts of the legs, in the area of the thighs, there are pockets with a flap that closes with a velcro-fiber tape system (an option with pockets on both sides is possible). Under the pockets, in the area of the knees, reinforcements from an additional layer of para aramid fabric are sewn, so that they form a pocket, the opening of which is located on the lower side, it is closed with the help of a velcro strip-fiber strip assembly and enables the insertion of knee protectors. On the legs of the pants, non-flammable reflective strips in gray and yellow colors, about 5 cm wide, are sewn around the entire circumference. - Specification of materials of suitable composition which are incorporated: - Basic material: - Waterproof and vapor-permeable two-layer laminated with a membrane, Surface weight: jacket approx. 220 g/m2, trousers approx. 245 g/m2 - Lining: - Jacket: Stitched lining knit/fabric, raw material composition Aramid 100% or simiral, Surface mass approx. 300 g/m2 - Pants: Aramid / Viscose FR, Surface mass approx. 125 g/m2 - Knee reinforcement: 100% p-Aramid knit - Reflective material: Reflective silver and fluorescent yellow tape, width 50 mm, tested degree of reflection after washing and burning - Auxiliary material: - Machine sewing thread: Zippers YKK: Metal; Velcro: non-flammable - Standards: SRPS EN 469:2010 (EN 469:2005), SRPS EN 13688:2015, (harmonized European standards: EN 13688:2013) or equivalent |  |  |  |
|  | FIREMAN'S BELT – 10 pcs. - Technical characteristics: - The fireman's belt consists of an 85 mm wide black belt. The belt has a rectangular buckle (muzzle) at one end, through which the other end of the belt is threaded. A buckle with two teeth is sewn to the running length of the belt belt, through which the other end with eyelets aligned next to each other is threaded - The lugs are used to hold the teeth of the buckle as well as to close and adjust the length of the belt - The belt that goes around the stomach has one eyelet on the side that has a raised leather plate as a support. It is possible to add a second holding eyelet symmetrically with the other - Attached to this retaining eye is a max. 770 mm long connecting device (including coupling and carabiner hook) made of twisted polyester cable fi 12mm - This tool has a carabiner hook with a multi-functional aluminum eyelet at its other end - The hook carabiner is a class T connector that can be locked with a Twistiock lock or a coil lock - An additional means of connection is secured by a stitched cover of black leather - Protection standard: DIN EN 358:2000 and DIN 14927:2005 or equivalent - Material: Fireman's belt made of black tape 85 mm wide, polyester rope 770 mm long with T type 5290 connector is installed in the belt |  |  |  |
|  | FIRE AX WITH CASE – 10 pcs. - Technical characteristics: - Red colored head with a rim that serves for piercing metal and the like, and on the other hand narrow cutting sharpened edge, blade - The handle is an oval tubular steel that is pressed into the head-blade - The handle is a non-slip rubber hand protection - Weight: 1000 - 1200 gr - Protective case for firefighter's axe, made of black leather with belt loop, specially adapted for the new axe with triangular blade, high-quality leather and workmanship - High-quality leather - Metal-reinforced corners |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >