# ANNEX II + III:	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Medical equipment	p 1 /…

Publication reference: RORS00270/Nova crnja/TD8

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Ultrasound machine​ for diagnostics – 1 piece General characteristics - Stationary digital color doppler ultrasonic dynamic apparatus range over 272dB, at with wheels , not portable concept, weighing up to 70 kg - Four active probe connectors , excluding the pencil connector . - Number of process channels minimum 270,000 - High-resolution LCD monitor minimum 21.5" on an artificial arm, with the ability to move and turn, as well as adjust the height. - Technically advanced minimum 14-bit digital Beam Former. - Management console with a minimum 10'' TFT Touch Screen panel or bigger , with possibility changes height . - Possibility programming shortcut layout to the TFT Touch Screen panel . - Maximum depth recording minimum 40 cm. - CINE memory in B mode minimum 10,000 frames . - Hard disk capacity minimum 500 GB. - Black and white digital thermal printer. Operating modes - B, M-mode, Color Doppler, Power Doppler, Pulsed Wave Doppler , CW Doppler, Triplex mode, display technique microvascularization with direction information ( not directional Power Doppler). - The possibility of displaying images in real time and "slow-motion" images next to each other at the same time , working precise heart function analysis muscles . Advanced functions - Automatic angle correction in PW doppler. - Spatial composition of the image with the possibility of adjusting the wave propagation angle. - Automatic Doppler spectrum outline in real time with by showing measurements. - Possibility upgrades on next advanced techniques: automatic NT measurement, to work with contrasting agents, automatic IMT measurement. - Program for enhanced visualization of the game during biopsy. - Automatic optimization images –Gain and TGC in B mode and band speed and base lines in PW mode. - Display trapezoidal image in case of use of linear probe. Probes - Convex abdominal probe, 1-5 MHz range or wider, with a minimum of 192 crystal elements. - Cardiological probe of the range 1-5 MHz, with a scanning angle of 90°. |  |  |  |
| 2 | Portable Biphasic Defibrillator for External Defibrillation, Synchronized Cardioversion, AED (Automated External Defibrillation), Pacing and Patient Monitoring with Carrying Handle - 2 Pieces General characteristics - TFT color display, diagonal min. 8.4" with the ability to display at least 4 curves and 5 numerical parameters - The device should have two sizes of pads for external defibrillation: for adults and for children - The shoes should have a built-in button for charging and discharging energy as well as selecting energy. - Ability to work on batteries, capacity of minimum 420 shocks with maximum defibrillation energy of 360J or minimum 10 hours of continuous monitoring - Battery charging time: maximum 3 hours, with grid power, up to 80% batteries must be charged in less than 2 hours - Level selection energy defibrillation : from 1 to 360J, 25 types , larger range impedance of 20-250 Ω, suitable for large number patients . - Speed charging from 0 to 200J: max 5 sec., with grid power supply or on full battery, up to 360J in less than 8 seconds - Fast recovery basic ECG lines after defibrillation: max 3 sec . - Automatic R tooth detection during synchronized cardioversion - Automatic internally discharge energy after expiration certain time period - Daily automatic testing of the battery and the entire device when grid powered, with memorization of the test results. Visual indicator of test results (correct/incorrect) on the front of the device - Visual and audible indication of energy charge - Setting upper and lower alarm limits for HR - Automatic detection of the pacing signal - Automatic detection of arrhythmias, with audible and visual alarms - The device has a switch with which the energy can be selected in steps, as well as the functions of monitoring, pacing, and AED. Energy can be selected in steps of 15, 20, 30, 50, 70, 100, 120, 150, 170, 200, 300, 360 - Memory of arrhythmias, memory of graphs and trend tables for 160h and memory of a minimum of 200 alarm events, with all relevant parameters and waveforms of 32s length. Storage of a minimum of 2000 NIBP measurements. Memory of a minimum of 5 complete 12-channel ECG reports, with diagnosis. Storage of a minimum of 480 minutes of taping, a maximum of 240 minutes of voice recording (up to 60 minutes for each patient) - Integrated 4-channel thermal printer, paper width minimum 80mm, printing speed 25/50mm/s, real-time printing 3.5s, 8s, 16s, 32s. - Operating temperature in the range from 0 to +45°C or more - Protection level IP44 or better - Weight of the device including one battery and pedals: maximum 8kg - Upgradeable for monitoring NIBP, IBP (minimum 2 channels), SpO2 and EtCO2, as well as 12-channel ECG with 6 waveforms - The device has hooks for hanging on the bed - The device can be connected to HIS, HL7, and central monitoring via wired and wireless means. - Required accessories: - 3-lead ECG cable, 1pc Thermal paper, 10pc User manual, 1pc |  |  |  |
| 3 | Dry sterilizer with natural circulation air – 3 pieces General characteristics - Volume- min 58 liters - The sterilizer made entirely of stainless steel - Digital adjustment of sterilization time and temperature - Fiberglass thermal insulation - Temperature range from 140o C to 250o C - Possibility of setting the sterilization time from 0 to 600 minutes and unlimited - The door capable to be locked with a key - Digital display - It has a visual and audible alarm for: excessive temperature; lower temperature than set; damaged temperature probe; damaged heating element. - Thermostat protection against excessive temperature - Maximum power 800W - Sterilizer has min 2 adjustable shelves - External dimensions 700mm x 480mm x 470mm ±5% - Internal dimension chambers 500mm x 360mm x 320mm ±5% - Required standards and directives - IEC EN 61010-1 or equivalent/similar - EEC 2006/95 or equivalent/similar - EEC 93/42 Medical Devices Directive or equivalent/similar - EEC 2004/108 or equivalent/similar |  |  |  |
| 4 | 12-channel ECG device - 3 pieces General characteristics - Minimum 7 inch LCD color screen on touch , with resolution min 800x480 - Alphanumeric keyboard - Grid and battery operated power supply . 130 views in battery mode - CardioTEKA software or appropriate/similar - Display on display all 12 drains in line with Einthoven, Goldberg, Wilson and Cabrera - modes : automatic , manual , automanual , rhythm , long - Formats presses : 3×4, 3×4+1R, 3×4+2R, 3×4+3R, 6×2, 6×2+1R, 6x2+2R, 6x2+3R, 12×1 - Frequency sampling rate : 8000 Hz - INOP signaling for each electrode - Display of battery charge - Detection pacemaker with sound signals - Automatic analysis and interpretation with by showing on screen - Sensitivity 2.5 / 5 / 10 / 20 mm/mV - Speed : 5 / 6.25 / 10 / 12.5 / 25 / 50 mm/s - Filters : - Grid filter: 50 Hz, 60 Hz, - Muscle filter: 25 Hz, 35 Hz, 45 Hz, - Vontour line filters: 0.15 Hz, 0.45 Hz, 0.75 Hz, 1.5 Hz, - Low-pass filter (LP): 75 Hz, 100 Hz, 125 Hz, 150 Hz, - Auto- adaptive filter (AA). - Adjustable length of recording automatic examination in an interval of 6-30 seconds - Reverse recording - Possibility upgrade with spirometry - Having an ECG function for examination on open heart - Having ECG- M function for sending examination by e- mail - Possibility of print on external printer on A4 format - Thermal printer with by using paper minimum width 112 mm - Possibility settings thickness curve for printing - Possibility settings parameters : speed , sensitivity , intensity printing - Integrated memory - minimum 1000 records - Communication : USB Host, USB, Ethernet - Dimensions : 350x245x130 mm ±5% - Weight max 1.5 kg - The device is supplied with next accessories: ECG cable EKG paper EKG electrodes extremities – complete Precordial ECG electrodes – set Gel Power cable​ |  |  |  |
| 5 | POCT analyzer device – 1set General characteristics FREND System - 1pc: - Method performance tests - immunofluorescence . - Required volume sample for performing D- Dimer , Procalcitonin and Cardiac panel (Troponin I, Myoglobin and CK-MB) – maximum 35μL for determination - Speed obtaining results 3 minutes to a maximum of 5 minutes - Possibility connections on the LIS. - Possibility memorization of up to >1000 results . - Possibility of quality control with specific Control cartridge . - Size of packages : 20 - 25 individually packaged tests - Possibility simultaneous obtaining emergency analysis – Troponin I, Myoglobin and CK-MB, for exclusion Heart attack myocardium - Dimensions appliances – maximum 240mm x 260mm x 175mm ±5% - Size screen - no smaller than 7.0 inches. - With the camera deliver external printer and automatic pipette Automated Pretreatment (AP) – 1 Piece - Dimensions appliances to maximum 195mm x 230mm x 210mm​ ±5% - Weight appliances up to 4 kg - Storage temperature range the appliance is 20-30 °C |  |  |  |
| 6 | Vitamin D Test - 400 Pieces Compatible with POCT Analyzer |  |  |  |
| 7 | Thyroid Duo Test - 400 Pieces Compatible with POCT Analyzer |  |  |  |
| 8 | PSA Plus Test - 400 Pieces Compatible with AP |  |  |  |
| 9 | Aneroid Sphygmomanometer - 10 pieces General characteristics - Cufflink washable , resistant​​ on wear and tear - Mesh filter in the valve return pressure provides protection from dust ingress - Ergonomic pump - Reliable, constantly adjustable air pressure in the discharge valve - Complete set: stethoscope, cuff and device case |  |  |  |