# ANNEX II + III:	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Specialized sewer breaking vehicle	p 1 /…

Publication reference: RORS00239/Municipality of Golubac/TD5

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Specialized sewer breaking vehicle: 1 piece Technical characteristics of the chassis: - Permissible total weight: maximum 7200 kg - Wheelbase: maximum 3800 mm - Front steering axle, load capacity: minimum 2,700 kg - Rear drive axle, dual wheels, load capacity: minimum 5,300 kg - Rear drive 4x2 - Chassis: "C" profile - truck chassis, minimum chassis profile thickness 5 mm - Front overhang: maximum 1060 mm - Diesel engine must meet at least EEC/EC Euro 6 standards - Engine displacement from 2900 to 3000 ccm - Engine power: minimum 125 kW - Torque: minimum 430 Nm - Radiator protection from the bottom - Manual transmission minimum, 6 forward gears and 1 reverse gear - Power drive/outlet from transmission in line with the requirements of the superstructure - Front and rear disc brakes - Anti-lock braking system (ABS) - Anti-skid regulation (ASR) - Electronic stability control (ESP) - Forward collision warning system and advanced emergency braking system - Emergency lane keeping system - Hill-start assist system - Drowsy driver warning - Speed ​​control and traffic sign recognition system - Forward-facing radar - Parking brake on the rear axle, mechanical control - Electric power steering, adjustable for tilt and height - System voltage 12 V - Alternator minimum 200 A - Battery minimum 100 Ah (12 V) - Audible signal when moving back - Semi-extended cabin, white or equivalent - Rear window on the cabin glass - Maximum number of seats 1+2 - Airbag for driver and passenger - All seats with headrests and seat belts - Audible signal for unfastened seat belt for driver and passenger - Electric windows for front doors - Vehicle keys - coded, in minimum 2 copies - Remote central locking - Main rearview mirrors heated and electrically adjustable - Digital instrument panel with a minimum 10-inch screen - Factory radio with a minimum 7-inch screen - Digital tachograph for two drivers - Connection point for forced towing of the vehicle forward - FMS connector - Automatic daytime running lights - Fog lights, front - Air conditioning - Cruise control - Tires 225/75R16 (6+1 spare) or larger - Spare wheel of standard dimensions - Tire pressure monitoring system - Fuel tank min. 100 liters - Minimum vehicle equipment: equipment in accordance with Serbian Traffic Law (first aid kit, spare bulb set, triangle, PP apparatus – S2, fluorescent vest, hydraulic jack of min. 2 t) and other if obligatory by law. - Wheel chocks – 2 pieces Technical characteristics of the superstructure: - Superstructure new and unused - Tank - Cylindrical tank. - Tank made of high-quality sheet metal ST52-3 or equivalent. - Tank sheet thickness minimum 5 mm - Tank capacity min. 3500 liters divided into two parts - Front part with a volume of min. 2,000 liters for clean water - Rear part min 1,500 liters for sludge and sewage content - Entire tank placed at an angle of 3° in relation to the chassis - On the upper part of the tank of the fecal part, installed a revision opening min. diameter Ø 500mm -At the back of the tank, installed door for cleaning sediment with a diameter of min.Ø 500mm -Opening for suction and discharge of pulpy material equipped with a ball valve min. DN80 – on the cleaning door - Protection the inside of the tank with an anti-corrosion coating - Breakwaters in the tank mounted that prevent water ripples during transport - The auxiliary frame made of U profile and according to the instructions of the chassis/undercarriage manufacturer - All external steel surfaces of the tank and auxiliary chassis sandblasted, thoroughly painted and finally varnished. - In the part where there is clean water, an overflow pipe for venting min 2" installed. - Visual control of the liquid level installed. - Tank can be filled with clean water through a special hydrant coupling located in the middle of the tank, min. type C, DN 52. - Automatic switch-off of the pump when the water runs out installed. - Warning orange rotating light on the rear of the vehicle installed. -On the sides of the tank, boxes for housing equipment installed. - VACUUM PUMP: - Minimum capacity 12,000 l/min - Air cooling of the vacuum pump - Maximum vacuum min. 95%, - Maximum over pressure min. 2 bars. - Vacuum pump drive: hydraulic motor - The vacuum system consists of the following parts: Ball level limiter, Precipitator, Suction filter, Suction silencer, Safety valve - pressure, Safety valve - vacuum, Exhaust silencer. - HIGH PRESSURE PUMP: - Minimum pressure 150 bars - Minimum flow 100 lit/min - Low water level alarm to prevent the pump from running dry -Water filter in front of the pump - High-pressure glycerin pressure gauge, resistant to freezing - Rear hose reel with hose diameter 3/4", length min. 80 m, maximum working pressure min. 250 bar, hydraulic winding. - High-pressure pump equipped with a suction filter, safety valve and operating valve. - Recirculation water heating system installed - COMMANDS IN THE CABIN: - Turning on the rotating light - Vacuum pump and high pressure pump operating hours meters - Start-stop for the high pressure pump and for the vacuum pump - CONTROL BOX REAR RIGHT SIDE: - The command to turn on the pumps - Alarm light for low water level - Counter of working hours - Shutdown in case of emergency - High pressure manometer, - Switching the high pressure pump on/off - Switching on/off the vacuum pump - Minimum water level indicator with audible warning - EQUIPMENT AND DOCUMENTATION THAT SHOULD BE DELIVERED WITH THE VEHICLE: - Key for clutches ABC -1 pc - Hydrant hose for filling the cistern - 1 pc - Gun with nozzle for washing - Channel nozzle for general cleaning of concrete pipes ¾" -Suction hose DN 80 x3 m - 4 pcs - Metal extension with coupling 1.5 m- 1 pc - When delivering the superstructure, the instructions for maintenance and handling of the superstructure and the catalog of spare parts must be delivered in the Serbian language IMPORTANT NOTE With the offer Tenderer should provide also following documents: - Catalogues, brochures, booklets, manufacturers’ declarations that sufficiently define the supplies being offered, and thus shall enable the Contracting Authority to check the information provided in the technical offer. - Technical description of the offered basic vehicle issued by the manufacturer or authorized seller/distributer (for the Country from where is Tenderer registered) of the basic vehicle, from which it can be clearly determined that all technical requirements have been fulfilled. The technical description must contain the number of the public procurement procedure, as well as the type and model of the basic vehicle offered. - Technical description of the offered superstructure, issued by the superstructure manufacturer, from which the fulfilment of all technical requirements can be clearly determined. The technical description must contain the number of the public procurement procedure, as well as the type and model of the superstructure offered - Warranty conditions must include location and contact for vehicle servicing provider including working hours. Servicing provider must be located in Republic of Serbia or if it is not it should provide feasible means of servicing for vehicle located in Serbia. |  |  |  |