# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Support field vehicles	p 1 /18

# Publication reference: RORS00005 – TD06/2

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

General Requirements

- All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

- The tenderer must provide the specifications of the offered items in the technical offer (Column 3.- Specifications offered), including details such as the manufacturer, product type, model and accurate characteristics of offered equipment for requested values. Additionally, the tenderer must submit brochure and catalogue documentation indicating the requested parameters of the offered vehicle and equipment. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

- If, in the document ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER -Column 3 (Specifications offered), the tenderer only provide generic confirmations such as "yes," "compliant," or text copied directly from Column 2, the offer may not be considered, unless accompanied by supporting documentation (e.g., a product catalog or similar). Tenderers must provide comprehensive information to demonstrate that the offered product meets all required specifications. If the details provided in the offered specifications or supporting documentation differ from the required specifications listed in Column 2, the offer will be deemed non-compliant.

- Documentary evidence must be provided for each item listed in the document ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER where compliance with the relevant standard is claimed.

- The selected contractor is required to provide all necessary documentation for the registration of the completed vehicle in accordance with the relevant legal regulations in the Republic of Serbia. This includes the Certificate from the Agency for Traffic Safety confirming the vehicle examination, as well as the technical inspection registration sheet, which is required for further vehicle registration.

- The tenderer must provide a list of authorized service centers for vehicles in the territory of the Republic of Serbia. If the tenderer does not have its own authorized service center, it is necessary to provide a contract(s) with an authorized service center (concluded at least 6 months before the announcement of this tender). Authorized service centers should provide 24/7 mobile support service.

- The tenderer is also required to supply manuals and other necessary documentation for the installation, operation, maintenance, and repair of the vehicles and equipment. These manuals should be provided in both English and Serbian languages, in both hardcopy and electronic formats.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| LOT 1 LOGISTIC VEHICLE All tenders submitted must comply with the requirements in the tender dossier and comprise: |  |  |  |  |
|  | LOGISTIC VEHICLE Quantity: 1 | Manufacturer’s name: Product/type, model: |  |  |
| 1. | Logistic vehicle for intervention in urban and rural areas. New, unused vehicle, the year of production 2025. or younger. Required technical characteristics: Engine power, drive, suspension and braking system 1. Diesel, 4 stroke, with direct fuel injection, turbo charged 2. Euro 5 standard minimum 3. Power min. 110 kW 4. Engine capacity from 1600 to 2000 ccm 5. Drive 4 x 4 6. Transmission automatized (forward + reverse) gears 7. Electric parking brake 8. Multifunctional steering wheel with height and depth adjustment 9. Cruise control 10. Hill assistance system 11. Disc brakes on front and rear axle 12. Collor: Red (Rall 3000 or equivalent) 13. Bumpers in vehicle color, or black Dimensions 14. Wheel base: min 3000 mm 15. Length: max 5150 mm 16. Height of the vehicle: max 2050 mm 17. Load capacity minimum 700 kg Steering system 18. Steering: Servo Cabin 19. Crew cab no. of seats min. 2+3 20. Driver seat: Adjustable 21. No. of doors: 4 on side (2 sliding doors)+ 2 on back part of vehicle Additional equipment 22. Additional equipment: - Tow hitch with capacity of minimum 2000 kg - Air conditioning - Rear camera - Tinted windows for passenger part of vehicle - Start – stop function - Electrically adjustable rear view mirrors - Multimedia system - Electric windows - Central lock, remote unlocking, - Wheel size minimum 16“ - Lifting jack -Tool set -Spare wheel -Flor mats front and rear -Mandatory equipment set (according to current regulations in RS) Electric installation 23. Voltage 12 V Fuel tank 24. Volume min 60 l |  |  |  |
| 2. | Delivery period is 6 months. Delivery address: Mije Kovačevića 2-4, 11000 Belgrade, Republic of Serbia. |  |  |  |
| 3. | LABELLING OF THE VEHICLE On the front sides of the vehicle cabin (front door, left and right side), the magnetic emblem of the Sector for Emergency Management, 30 cm high and 30 cm wide, shall be placed. If these recommendations can’t be followed, the dimensions of the emblem must be proportional to the dimensions of the vehicle on which they are installed and approved by contracting authority. Vehicle shall also be visibly labelled as per EC visibility requirements (according to the latest Communication and Visibility Requirements for EU-funded external action, laid down and published by the European Commission website). Vehicle must be marked with durable/UV resistant plastic stickers with above mentioned content. The selected bidder shall produce a draft layout for written approval by Contracting Authority prior to production. |  |  |  |
| 4. | The warranty period for vehicle is minimum 24 months. During the warranty period, any malfunction of the products covered by the warranty will be solved free of charge, without additional costs to the contracting authority. |  |  |  |

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| LOT 2 COMMAND AND RECONNAISSANCE VEHICLE WITH UAV All tenders submitted must comply with the requirements in the tender dossier and comprise: |  |  |  |  |
|  | COMMAND AND RECONNAISSANCE VEHICLE WITH UAV Quantity: 1 | Manufacturer’s name: Product/type, model: |  |  |
| 1. | Pick-up vehicle for all types of emergencies, capable for operations in rural areas and hardly accesible terains. New, unused vehicle, the year of production 2025. or younger. Required technical characteristics: Engine power, drive, suspension and barking system - Diesel, 4 stroke, with direct fuel injection, turbo charged - Euro 6 standard - Volume min. 2.4 l - Power min 100 kW - Engine torque min. 400 Nm. - Drive 4 x 4 - Transmission manual or automatic with reduction and rear differential lock - Rear leaf suspension - Brakes with ABS, electronic stability control, traction control and hill assistance system - Front and rear parking sensors - Wheel size min. 17” Dimensions - Length: max 5500mm - Approach angle min 29⁰ - Departure angle min 27⁰ - Ramp angle min 23⁰ - Clearance min 300mm - Fordability min 700 mm Cabin - Double cab no. of seats min. 4 - No. of doors: 4 - Multi-function steering wheel - Automatic air condition - Multi-function colour display min 8” - Navigation - Air bags min. for driver and co-driver, driver knees and side air bags Additional equipment - Additional equipment - front fog lights - electrically adjustable rear view mirrors with a heater - electric windows - central lock, remote unlocking, -tools for spare wheel change -spare wheel in full size -mandatory equipment set (according to current regulations in RS) -rubber mats Fuel tank - Volume min 70 l Cargo space - Hard top cargo cover for the protection of equipment from dust and rain with minimum one opening (on the rear side of the vehicle) - Sliding deck placed inside cargo space for easier approach and utilization of the equipment. Deck load capacity min 200 kg - Display mount suitable for carrying ≈32” TV or computer monitor. Display mount should have capability to be positioned in transport and operational mode. During operational mode it should hold TV or computer monitor in working position inside the cargo space (easy approach and watching from the rear side of the vehicle) - Emergency lighting system: blinking blue lights modules installed in the front grill of the vehicle and rear part of the vehicle - Audio signalization: Audio signalization with sweeping horn, PA system (100 W horn), dashboard in the cabin Power supply equipment - Power generator -Monophasic inverter power generator -Minimal power 1800 W -Voltage 230 V -Power sockets: min. 2 x sockets 230V AC and 1 x 12V DC -Weight of empty generator max 25 kg - Battery power station: -Lithium Iron phosphate batteries min. capacity 2800 Wh -true sine wave output: THD<3% -protection: short circuit protection, temperature control, overvoltage, overload and overcurrent -Outputs: min 1 X USB A, 2 X USB A QC with 5V and 9V, 2 X DC 12V and 2 X 230V - Maximum weight 40 kg - Extension power cable reel with min 30 m length and min 3 X 230V sockets. |  |  |  |
| 2. | Unmanned Aerial System - UAV -Type: VTOL multirotor battery powered UAV - dimensions max: 1000 mm x 800 mm x 500 mm and MTOV 25kg - Flight speed min: 20m/s -Operational celling min 5000 AMSL -Operational distance min 20 km -Flight autonomy min 55 minutes (with adequate payload) -Hovering precision: ± 0,1 m horizontal and ± 0,1 m vertical -Wind resistance: min 12m/s - Dust and water protection: min. IP55 - Operational temperature range: min -20 to +50 ° С -High precision obstacle sensing system onboard -Remote ID onboard -Possibility of connecting at least 2 different payloads simultaneously -Control station with min 7” display and min 1000 nits brightness -UAV system is delivered with adequate battery charging station and min of 3 sets of batteries. -UAV system is delivered with high precision positioning module supporting GPS, GALILEO and GLONASS satellite positioning systems. Module is used for high precision mapping and as a signal transmission relay for the UAV. Module is delivered with a mounting pole and a tripod. - UAV Payload No. 1 -Hybrid payload with max. weight 1 kg -Water and dust protection min. IP 54 -Operational temperature min from -20 to +50 C -Zoom camera with min 40 MP and min. hybrid zoom of 32 x, and min digital zoom 400 x -Wide angle camera with min 40 MP and min 75⁰ DFOV -Infrared thermal camera: micro bolometer, with digital zoom min 30x, video resolution min 1280 x 1024, temperature measurement range from min 0 to 600⁰ C and different palate display -Laser range finder for up to 3000 m -NIR auxiliary light Payload is delivered with adequate micro SD card with min capacity of 128 GB - UAV payload No. 2 -Hybrid LiDAR/RGB camera sensor max weight 1,6 kg -Water and dust protection min IP 54 -Operational temperature min from -20 to +50 C UAV software - Analysis software -Permanent, offline software licence -High precision 2D and 3D modelling and mapping software which use aerial data to create maps and models for analysis and decision making in floods, wildfires, earthquakes, landslides etc. -2D and 3D reconstruction from RGB photos -3D modelling by processing LiDAR data -Multispectral analysis and creation of vegetation index maps -Software must be compatible with offered UAV, sensors and high precision positioning module. UAV training - Theoretical training The theoretical training shall be organized in accordance with the “UAV Training Plan and Program” officially approved by the Ministry of Interior (MoI). It must, at a minimum, cover the following subjects: - Aviation law - Meteorology - Principles of flight - Aircraft general knowledge - Navigation - Human performance and limitations - Flight performance and planning - Communication The training will be provided for two (2) SEM members by an Authorized Training Organization (ATO). Upon successful completion, participants will receive certificates approved by the National Aviation Authority. The training will be conducted in Serbian language, and the date and time will be determined later by mutual agreement between the client and the training organization. - Practical training The practical training shall be organized in accordance with the “UAV Training Plan and Program” officially approved by the Ministry of Interior (MoI). It must, at a minimum, include the following topics: - Basic operation of the UAV, corresponding to the delivered UAV type - Basic operation of cameras and sensors, corresponding to the delivered equipment - Basic flight skills and procedures related to the delivered UAV type The training will be provided for two (2) SEM members by a Certified Training Instructor (holding a certificate issued by the UAV manufacturer). Upon successful completion, participants will receive certificates of training completion. The training will be conducted in Serbian language, and the date and time will be determined later by mutual agreement between the client and the training organization. |  |  |  |
| 3. | IT and communication equipment - Laptop computer - Min. 15” display, FHD IPS anti-glare 300 nits - Min. Intel Core i7 processor, 14C with 4.9GHz or equivalent - Min. 32 GB SO-DIMM DDR5 - Min. 1 TB SSD NVMe - Min. NVIDIA RTX 5060 8GB GDDR7 graphic card or equivalent - Min. 2 x USB-A, 1 x USB-C, 1 x HDMI, 1 x, 1 x Ethernet (RJ 45) - Desktop computer - Min. Intel Core i7 processor with 3.4GHz or equivalent - Min. Min. 64 GB DIMM DDR5 - Min. 2 TB SSD NVMe - Min. NVIDIA RTX 3060 12GB GDDR graphic card or equivalent - Min. 750 w power supply - Min. 2 x USB-A, 1 x USB-C, 1 x Ethernet (RJ 45) - Min 19” display, keyboard and mouse - HDMI extender -Wireless HDMI extender “plug and play” kit with min of 30m transmitting range - 4G modem - Dual band 4G LTE router with SIM card slot and wireless standards IEEE 802.11ac/n/a 5GHz, IEEE 802.11b/g/n 2.4GHz - IPS ≈ 32” anti-glare monitor, with minimum full HD resolution, 400 nits and 1000:1 contrast and 1xHDMI, 1xUSB C and 1xUSB A connector. Monitor is delivered with mount adequate to the size and weight of the monitor and protective case. Case is made of rugged plastic material and it is filed with foam/sponge like fabric in order to protect monitor during transportation. - External data storage with min 2 TB capacity, USB C connector and min 1000 MB/s reading and writing speed. |  |  |  |
| 4. | Colour: Vehicle should be delivered in Red RAL 3000 or White RAL 9010 colour. |  |  |  |
| 5. | Delivery period is 6 months. Delivery address: Mije Kovačevića 2-4, 11000 Belgrade, Republic of Serbia. |  |  |  |
| 6. | LABELLING OF THE VEHICLES On the front sides of the vehicle cabin (front door, left and right side) the emblem of the Sector for Emergency Management, 30 cm high and 30 cm wide, shall be placed, as well as the emergency service number "193", which is placed just below the emblem of the Sector for Emergency Management. The height of a number is 15 cm, and the width is 2.5 cm. In case the vehicle body does not allow such marking, it is allowed to place the call number centred on the rear side door of the vehicle body. If these recommendations can’t be followed, the dimensions of the emblem and the numbers must be proportional to the dimensions of the vehicle on which they are installed and approved by Contracting authority. Vehicle shall also be visibly labelled as per EC visibility requirements (according to the latest Communication and Visibility Requirements for EU-funded external action, laid down and published by the European Commission website). Each vehicle must be marked with durable/UV resistant plastic stickers with above mentioned content. The selected bidder shall produce a draft layout for written approval by Contracting Authority prior to production. |  |  |  |
| 7. | Warranty period is minimum 5 years for the vehicle and minimum 2 years for the equipment. During the warranty period, any malfunction of the products covered by the warranty will be solved free of charge, without additional costs to the contracting authority. |  |  |  |