# VOLUME 3

# TECHNICAL SPECIFICATIONS

The location of the renovation works is in Timisoara, B-dul Revolutiei din 1989 no. 8, the ground floor of the building in the yard. The building is owned by the Bastion-Varbastya Association. It consists of a ground floor and one upper floor and is located in the inner courtyard. The renovation works will be carried out on the ground floor at the back part of the building, where there are currently industrial rooms as follows: storage room 1 – 39 sqm, storage room 2 – 19 sqm, storage room 3 – 50 sqm, storage room 4 – 32,5 sqm, hallway – 33,40 sqm, two toilets.

After renovation, there will be a conference room (39 sqm), a reception area (19 sqm), a training room (50 sqm), a co-working space (32,5 sqm), hallway and two toilets. Only renovation works will be carried out, with no structural modifications.

The following works will be performed:

- Removal of plaster from all walls

- Removal of existing floor finishes

- Dismantling of doors and windows

- Demolition of the 12 cm non-structural partition wall between the sanitary group and the corridor

- Reinstatement of plaster, skim finishing, and application of washable paint to all walls and ceilings

- Replacement of all existing windows with new engineered wood windows fitted with double-glazed units

- Interior doors shall be manufactured from engineered laminated wood, while exterior doors shall be metal.

- Execution of floor base layer

- Execution of ceramic floor finishes in the reception, hallway, and sanitary areas

- Execution of heavy-duty laminate floor finishes in the training room, conference room, and co-working space

- Execution of ceramic wall tile cladding in the sanitary areas to a height of 2.00 m

- Supply and installation of HPL sanitary partition panels in the restroom areas

- Equipping of sanitary facilities with fixtures and accessories

- Renewal of electrical and lighting installations,

For detail see VOLUME 4.2.3 — BREAKDOWN OF THE LUMP-SUM PRICE and the attached drawings and DALI.