# VOLUME 3

TECHNICAL SPECIFICATIONS

FACILITY: Health center "Srpska Crnja"

HOLDERS OF RIGHTS TO THE OBJECT: Municipality of Nova Crnja, JNA Street 110

Health center "Srpska Crnja" - user

FACILITY ADDRESS: Srpska Crnja, ulica Patrijarha Arsenija Čarnojevica no. 15, 23220 Srpska Crnja

LOCATION :

Health care building, Dom zdravlja "Srpska Crnja", number 1, floors P+1+Pot, is located in Srpska Crnja, in Patrijarha Arsenija Čarnojevica street number 15, on cadastral plot number 2365, registered in the real estate register number 378 ko Srpska Crnja . The area of the plot is PKP= 1580.00 m 2 . The plot is in the shape of the Latin letter L. Building number 1 is rectangular in shape at the base, built-up area P1= 611.00 m 2 . The purpose of building number 1 is a building for the provision of health services .

SPATIAL CONCEPT:

The object is located in the central part of the settlement, in the city center, within the city center of Srpska Crnja, in Patrijarha Arsenija Čarnojevica street number 15. The object is detached , 4.00 m away from the regulation line. Basically, the building is rectangular in shape, with the orientation of the ridge in the northwest-southeast direction. The main entrance to the building is from Patrijarha Arsenija Čarnojevica street .

CONSTRUCTIVE SYSTEM:

The constructive system is reinforced concrete: a system of AB columns, connected by bolts in two orthogonal directions. The filling is made of solid brick with a thickness of d=25 cm, and the partition walls are made of solid brick with a thickness of d=12 cm. The walls are plastered on both sides: internal, with lime plaster and external, with extension plaster. The mezzanine construction is a TM ceiling with a pressed AB panel. The roof construction is wooden, mansard roof system, floored and covered with flat galvanized, plastic sheeting. The staircase is reinforced concrete.

Water is drained from the roof through horizontal and vertical galvanized gutters. The interior walls and ceilings are plastered and painted with semi-dispersion paint. The walls in the bathroom are lined with ceramic tiles. The floors are covered with granite slabs.

Carpentry: internal doors are made of carpentry, single-leaf, solid, protected with oil paint. The entrance door is Al lock, glazed with flat glass, and the windows are made by carpenters, properly ironed, glazed with flat glass. The windows are scattered, they do not seal, which causes the appearance of "draughts" and the impossibility of good heating of the object. The wooden joinery has a visible lack of putty at the junction of the glass and the wooden frame.

The eaves on the outside of the window are made of galvanized sheet , damaged and do not adhere to the edges of the walls, which causes atmospheric water to flow along the parapet under the window, and it is necessary to replace the facade carpentry. The residential and commercial space is equipped with the following installations:

- Water supply and sewerage

- Electrical installation

- Central heating